Andipatti Animal Feed System

Document 09 — Complete Hardware Integration Specification v1.0

Implementation-ready specification • Flutter Edge Device • Go Backend • Docker • Offline-first • POS-integrated

Document sequence: 09 of 20 | No sprint planning

This document defines the hardware boundary for barcode scanners, digital weighing scales, ESC/POS printers, cash drawers, customer/payment displays and related POS peripherals. Hardware is an edge concern: Bluetooth/USB/RS-232 device access stays on the Flutter device, while authoritative business state remains in the Go/PostgreSQL platform.

# 1. Document Control

| Item | Specification |
| --- | --- |
| Document ID | AAF-HW-009 |
| Version | 1.0 |
| Status | Implementation Baseline |
| Primary consumers | Flutter, Backend, QA, Operations, Hardware/IT |
| Authoritative dependencies | Documents 01–08 |
| Hardware principle | Hardware failures must never corrupt finalized financial/inventory state. |
| Deployment | Backend/support services containerized with Docker; physical peripherals attach to POS devices. |

# 2. Objectives

Provide a vendor-neutral hardware abstraction layer.

Support USB, Bluetooth/BLE and RS-232 serial peripherals where appropriate.

Support barcode scanners as keyboard-HID and serial devices.

Support digital weighing scales with stable-weight validation.

Support ESC/POS thermal printers and common connection modes.

Support cash-drawer triggering through printer RJ-12/RJ-11 kick interface where available.

Support optional customer-facing display and payment/QR display.

Persist pairing/configuration per device and tenant.

Provide diagnostics, health state, reconnection and controlled fallback.

Keep hardware I/O outside authoritative accounting/inventory transactions.

Make all device actions observable and auditable without logging secrets.

# 3. Non-Goals

Do not run Bluetooth/USB hardware drivers inside backend Docker containers.

Do not make a printer, scale or scanner a prerequisite for server-side invoice finalization unless a specific business rule explicitly requires the scale for a weighted item.

Do not store raw payment credentials in hardware adapters.

Do not infer weight, payment success or print success from UI state alone.

Do not silently switch hardware devices without an explicit pairing/configuration policy.

# 4. Hardware Architecture

Flutter POS
  ├── Hardware Manager
  │    ├── Scanner Adapter
  │    ├── Scale Adapter
  │    ├── Printer Adapter
  │    ├── Cash Drawer Adapter
  │    ├── Customer Display Adapter
  │    └── Payment Display/QR Adapter
  │
  ├── Device Registry / Pairing State
  ├── Local Diagnostics
  └── SQLCipher Local State
          |
          | HTTPS / WSS
          v
      Go API / Workers
          |
      PostgreSQL / Redis(optional) / R2

The Flutter hardware layer exposes stable domain interfaces. Vendor/protocol-specific code is isolated behind adapters so the POS engine is not coupled to a particular manufacturer.

# 5. Hardware Device Registry

| Field | Requirement |
| --- | --- |
| device_id | Globally unique application device identity. |
| tenant_id | Mandatory tenant ownership. |
| pos_terminal_id | Logical POS terminal association. |
| hardware_type | SCANNER / SCALE / PRINTER / CASH_DRAWER / DISPLAY / PAYMENT_DISPLAY. |
| connection_type | USB / BLUETOOTH / BLE / RS232 / NETWORK / HID. |
| vendor | Manufacturer, if known. |
| model | Model identifier, if known. |
| serial_number | Captured where available. |
| firmware_version | Captured where available. |
| configuration | Versioned non-secret configuration. |
| status | PAIRED / CONNECTED / DISCONNECTED / ERROR / DISABLED / REVOKED. |
| last_seen_at | Last successful communication. |
| paired_at | Pairing timestamp. |
| revoked_at | Revocation timestamp when applicable. |

Hardware configuration changes require authorization.

Device pairing must be tenant-scoped.

Revoked POS devices must not perform authoritative financial operations.

Sensitive Bluetooth/USB permissions remain OS-controlled.

# 6. Hardware Capability Matrix

| Hardware | Required? | Primary connection | Failure fallback |
| --- | --- | --- | --- |
| Barcode scanner | Recommended | USB HID / Bluetooth / Serial | Manual/search input |
| Digital scale | Required for weighted workflows | RS-232 / USB serial / Bluetooth | Authorized manual weight |
| Thermal printer | Recommended | USB / Bluetooth / Serial / Network | Reprint/WhatsApp/manual receipt |
| Cash drawer | Optional | Printer kick / USB | Manual drawer operation with audit |
| Customer display | Optional | USB/Serial/Network | POS screen |
| Payment/QR display | Optional | USB/Network/Bluetooth | Flutter screen QR |

# 7. Common Connection State Machine

| State | Meaning |
| --- | --- |
| DISCOVERING | Searching for configured/available device. |
| PAIRING | OS/application pairing in progress. |
| CONNECTING | Transport connection being established. |
| CONNECTED | Ready for commands/data. |
| DEGRADED | Connected but protocol/read/write errors observed. |
| DISCONNECTED | No active transport. |
| RECONNECTING | Automatic bounded retry. |
| ERROR | Persistent device/protocol/configuration error. |
| DISABLED | Temporarily disabled by operator/admin. |
| REVOKED | Application identity revoked; cannot be used. |

Automatic reconnect must use bounded exponential backoff.

Reconnect attempts must not freeze the POS UI.

Only one active adapter instance may own a physical device per terminal.

Connection events are observable and locally visible to authorized operators.

# 8. Barcode Scanner Integration

## 8.1 Supported modes

USB HID keyboard-wedge.

Bluetooth HID.

Serial/COM scanner where supported.

Vendor SDK only behind a scanner adapter when generic protocols are insufficient.

## 8.2 Scan processing

Receive scanner input.

Detect configured terminator/prefix/suffix.

Normalize whitespace and control characters.

Validate barcode length/format according to product configuration.

Resolve exact barcode locally or through API.

Present product selection to POS.

Apply configured duplicate-scan behavior.

Record device and timestamp for diagnostics.

## 8.3 Duplicate scans

Rapid identical scans may intentionally increase quantity for countable products.

Weighted/loose products must follow product-specific scan behavior and must not multiply a scale reading unexpectedly.

Configurable debounce interval prevents accidental repeated input.

Manual correction remains available.

## 8.4 Unknown barcode

Display explicit unknown-barcode status.

Allow search by Tamil/English/alias/product code.

Never auto-create a product from scanner input.

Authorized users may open product-master workflow.

# 9. Digital Weighing Scale

## 9.1 Supported transports

RS-232 serial.

USB-to-serial adapters presenting a serial interface.

Bluetooth serial profile where supported by the device.

BLE only when a documented vendor protocol exists.

# 9.2 Configuration

| Parameter | Purpose |
| --- | --- |
| baud_rate | Serial speed. |
| data_bits | Usually 7/8 according to scale protocol. |
| parity | NONE/ODD/EVEN. |
| stop_bits | 1/2. |
| read_mode | CONTINUOUS / REQUEST_RESPONSE. |
| unit | kg/g or configured unit. |
| stability_duration_ms | Minimum stable period. |
| allowed_variance | Maximum permitted variation. |
| zero_tolerance | Acceptable near-zero range. |
| max_weight | Safety/business limit. |
| protocol_profile | Vendor/protocol decoder identifier. |

Defaults must be configured per hardware model/protocol. Never assume a universal serial configuration.

# 9.3 Scale reading pipeline

RAW BYTES
 -> FRAME DETECTION
 -> CHECKSUM/VALIDATION (if protocol supports)
 -> PARSE SIGN + VALUE + UNIT + STATUS
 -> UNIT CONVERSION
 -> RANGE VALIDATION
 -> STABILITY FILTER
 -> STABLE WEIGHT EVENT
 -> POS WEIGHT SERVICE
 -> CART LINE
 -> FINALIZATION REVALIDATION

## 9.4 Stable-weight rules

A stable reading requires the configured duration and variance criteria.

Scale status flags such as overload, motion, negative or error must be rejected.

Zero/negative readings cannot finalize a normal positive-weight sale.

Automatic acceptance must use a stable event, not an arbitrary latest frame.

Manual weight entry is permission-controlled and audited.

At finalization, the backend validates the quantity against product/UOM rules; it does not trust a client-side stability flag as a financial authority.

# 10. Scale Safety & Tare

Gross, tare and net concepts must remain distinct where the weighing workflow uses them.

Where procurement/GRN uses sack tare, the GRN engine validates tare separately from POS.

POS weighted sales generally consume net sale quantity.

Scale overload must immediately stop automatic capture.

An operator must not be able to enter a negative net weight.

Any manual tare adjustment is explicitly recorded and permission-controlled.

# 11. Scale Protocol Adapter Contract

interface ScaleAdapter {
  connect(config) -> Result
  disconnect() -> Result
  readLive() -> Stream<ScaleReading>
  requestStableReading() -> Result<StableWeight>
  zero() -> Result
  tare() -> Result
  diagnostics() -> Result<ScaleDiagnostics>
}

The exact Dart interface may differ, but the semantic contract must remain equivalent. Backend services never directly depend on the physical protocol.

# 12. Thermal Printer / ESC-POS

## 12.1 Supported transports

USB serial/USB printer interface.

Bluetooth serial where supported.

RS-232/serial.

Network/TCP printing where the printer supports it.

## 12.2 Printer responsibilities

Render finalized receipt content into an ESC/POS command stream.

Handle paper width and character capability.

Print Tamil through supported font/bitmap/image rendering where native code-page support is unavailable.

Support barcode/QR printing when printer supports it.

Support cash-drawer kick where configured.

Return print status/errors where the transport provides them.

## 12.3 Print pipeline

Retrieve finalized invoice receipt snapshot.

Resolve printer profile.

Render layout.

Encode Tamil/Unicode using configured strategy.

Generate ESC/POS command stream.

Open transport.

Write with bounded timeout.

Flush/close.

Record print attempt.

Expose success/failure to UI.

Printing occurs after financial commit. A print exception must never roll back the invoice.

# 13. Tamil Printing

Preferred method: printer-supported Unicode/font rendering.

Fallback: render Tamil receipt text as raster image/bitmap if printer cannot render required Unicode.

Printer profile stores supported character strategy.

Golden receipt tests must verify Tamil output on each supported printer profile.

Do not silently replace Tamil with unrelated glyphs.

If Tamil cannot be printed reliably, offer WhatsApp/PDF/digital receipt fallback according to configured capabilities.

# 14. Receipt Layout Requirements

| Section | Required content |
| --- | --- |
| Header | Shop name, address/contact and configured identifiers. |
| Invoice | Invoice number, date/time, financial year/series as applicable. |
| Customer | Customer name/phone or Walk-In according to privacy policy. |
| Lines | Product, UOM, quantity/weight, rate, discount, amount. |
| Tax | Tax breakup when applicable. |
| Tender | Cash/UPI/bank/Khata/split details and relevant reference. |
| Totals | Subtotal, discount, tax, rounding, grand total, change/balance. |
| Footer | Configured message, return policy and digital receipt option where enabled. |

# 15. Reprint & Print Retry

Reprint is always based on finalized invoice data.

Reprint permission is role-controlled.

Each reprint creates an audit event.

Retry is safe because printing is an external side effect.

Print attempt IDs prevent duplicate operational records but do not prevent legitimate reprints.

Printer recovery should allow retry without reopening or editing the invoice.

# 16. Cash Drawer

Preferred trigger: ESC/POS cash-drawer kick from printer.

Drawer opening occurs only for configured cash operations.

Opening event records user, terminal, timestamp and reason where applicable.

Drawer-open failure does not alter invoice/payment state.

Physical cash reconciliation remains part of cash session/EOD.

Do not expose drawer control to unauthorized roles.

ESC/POS concept:
ESC p m t1 t2
(Exact bytes depend on printer and configured kick parameters.)

# 17. Customer Display

Show cart lines, quantities, price, discount and total.

Show tax only as configured.

Show payment amount/QR when payment workflow requires it.

Do not display sensitive Khata exposure, internal cost, margin or owner-only data.

Display connection loss without blocking the POS workflow.

# 18. Payment/QR Display

QR content originates from the payment provider/payment intent, not from manually typed arbitrary data.

Display may be a second monitor, network display, serial display or Flutter screen.

Payment display is informational; provider verification remains authoritative.

Do not mark payment successful from QR display interaction.

# 19. Hardware Pairing & Provisioning

Admin identifies terminal.

Discover or enter hardware.

Select hardware type and protocol profile.

Pair/connect through supported OS mechanism.

Run diagnostics test.

Save configuration.

Record device identity.

Run test scan/weight/print.

Mark device ACTIVE.

Provisioning must validate tenant and terminal ownership.

Device configuration changes are audited.

Secrets/credentials, if any, use secure storage or environment/secret-management mechanisms.

# 20. Diagnostics Mode

| Diagnostic test | Expected result |
| --- | --- |
| Scanner test | Decoded scan displayed; no sale created. |
| Scale test | Raw and normalized reading displayed; no stock impact. |
| Printer test | Test receipt printed; no invoice created. |
| Drawer test | Drawer kick executed if supported; audited. |
| Customer display test | Test content rendered. |
| Payment display test | Test QR/display content rendered without payment state change. |
| Connectivity test | Transport latency/error shown. |
| Protocol test | Parser/checksum validation shown where available. |

Diagnostic mode must never create inventory, accounting, payment or invoice transactions.

# 21. Hardware Health Model

| Metric | Examples |
| --- | --- |
| Connectivity | connected/disconnected duration |
| Latency | read/write/print latency |
| Errors | protocol, transport, paper, checksum, overload |
| Availability | successful operations / attempted operations |
| Reconnects | count and last reconnect |
| Firmware | version where discoverable |
| Paper | low/out if printer reports it |
| Scale state | stable/motion/overload/error |
| Scanner | last scan timestamp |

Health data should be retained according to operational observability/retention policy and should not grow without bounds.

# 22. Failure Isolation Rules

Hardware adapter exceptions are caught at the edge boundary.

A printer exception never rolls back an invoice.

A scanner failure never blocks manual search.

A scale failure only blocks automatic weighted capture; authorized manual entry may be allowed.

Customer-display failure never blocks billing.

Payment-display failure never changes payment state.

Hardware reconnect runs asynchronously.

Hardware event streams must be cancelled on screen/device lifecycle changes to avoid leaks.

# 23. Offline Hardware Operation

Scanner works against local SQLCipher product index.

Scale can capture weights offline.

Printer can print local finalized/provisional receipts according to offline receipt policy.

Hardware events can be stored with local transaction envelope IDs.

On reconnect, hardware telemetry does not override authoritative financial data.

Offline provisional receipt must be clearly distinguishable if local policy requires it.

# 24. Hardware + POS Transaction Boundary

Hardware event
   -> local POS state
   -> user confirmation
   -> POS quote
   -> payment workflow
   -> server finalization
   -> committed invoice
   -> post-commit print/share

Never:
hardware event -> direct DB financial write
printer success -> invoice finalization
QR display -> payment success
scale "stable" flag -> unconditional stock decrement

# 25. Backend Hardware APIs

| Endpoint | Purpose |
| --- | --- |
| GET /api/v1/devices | List authorized POS devices/hardware. |
| POST /api/v1/devices | Register/configure logical device where permitted. |
| POST /api/v1/devices/{id}/pair | Pair/provision workflow metadata. |
| POST /api/v1/devices/{id}/revoke | Revoke device. |
| GET /api/v1/devices/{id}/health | Health/diagnostics summary. |
| POST /api/v1/receipts/{invoice_id}/print | Create controlled print request if remote print orchestration is used. |
| GET /api/v1/sales/invoices/{id}/receipt | Retrieve receipt representation. |
| WSS /api/v1/ws | Optional payment/status and operational event channel. |

Exact routes/schema are governed by Document 02. Direct physical I/O remains on Flutter.

# 26. Database Mapping

| Concern | Entities from Document 01 |
| --- | --- |
| Logical POS/device | devices; device_sessions |
| Audit | audit_logs |
| Receipt/notification | notification_jobs; message_deliveries |
| Invoice source | sales_invoices; sales_invoice_lines; invoice_tax_lines |
| Payment | payment_intents; payments; payment_webhook_events |
| Outbox | integration_events |
| Offline hardware event | sync_transactions / local SQLCipher tables |

If the implementation needs a richer hardware inventory table, it must remain tenant-scoped and be added through a controlled schema revision rather than hidden application state.

# 27. Flutter Architecture Mapping

lib/
  features/hardware/
    domain/
      hardware_device.dart
      hardware_capability.dart
      scale_reading.dart
      printer_job.dart
    application/
      hardware_manager.dart
      device_pairing_service.dart
      diagnostics_service.dart
    infrastructure/
      scanner/
        hid_scanner_adapter.dart
        bluetooth_scanner_adapter.dart
        serial_scanner_adapter.dart
      scale/
        rs232_scale_adapter.dart
        bluetooth_scale_adapter.dart
        protocol_profiles/
      printer/
        escpos_renderer.dart
        usb_printer_adapter.dart
        bluetooth_printer_adapter.dart
        serial_printer_adapter.dart
      drawer/
      display/
    presentation/
      hardware_settings/
      diagnostics/

# 28. Go Backend Mapping

DeviceService: device registration, status, revocation and authorization.

AuditService: pairing/configuration/diagnostic audit.

ReceiptService: immutable receipt data model.

Notification/MessageService: WhatsApp fallback.

PaymentService: payment intent/status authority.

SyncService: offline envelope reconciliation.

Ops/HealthService: backend-visible device/terminal health where applicable.

The backend should not implement vendor serial/Bluetooth drivers. It consumes logical device state, transaction metadata and trusted API calls from the Flutter edge.

# 29. Security

Only authenticated, authorized users can configure hardware.

Device IDs are authenticated and tenant-bound.

Revoked devices are rejected server-side.

USB/Bluetooth permissions follow OS security model.

Hardware debug logs must not expose secrets.

Payment QR payloads are treated as payment data and are not logged in full unless explicitly required and protected.

Receipt data follows privacy policy; avoid unnecessary PII in diagnostics.

Admin-only diagnostics may expose raw protocol data where operationally necessary, with controlled retention.

Firmware update capability, if later added, must use signed/verified firmware and a separate security specification.

# 30. Hardware Role Matrix

| Operation | Cashier | Senior Cashier | Manager/Owner | Admin/IT |
| --- | --- | --- | --- | --- |
| Use scanner | ✓ | ✓ | ✓ | ✓ |
| Use scale | ✓ | ✓ | ✓ | ✓ |
| Manual weight | Configured | ✓ | ✓ | ✓ |
| Print/retry | ✓ | ✓ | ✓ | ✓ |
| Reprint | ✓/audit | ✓ | ✓ | ✓ |
| Drawer open | Configured | ✓ | ✓ | ✓ |
| Pair device | — | — | ✓ | ✓ |
| Change protocol config | — | — | Limited | ✓ |
| Run diagnostics | Basic | ✓ | ✓ | ✓ |
| Revoke device | — | — | ✓ | ✓ |

# 31. Error Catalogue

| Code | Meaning |
| --- | --- |
| HW_NOT_FOUND | Configured hardware unavailable. |
| HW_NOT_PAIRED | Pairing required. |
| HW_PERMISSION_DENIED | OS permission denied. |
| HW_CONNECTION_FAILED | Transport connection failed. |
| HW_TIMEOUT | Operation timed out. |
| HW_PROTOCOL_ERROR | Protocol/parser error. |
| HW_CHECKSUM_ERROR | Invalid protocol checksum. |
| HW_UNSUPPORTED | Capability/protocol unsupported. |
| SCALE_UNSTABLE | Reading not stable. |
| SCALE_OVERLOAD | Scale overload. |
| SCALE_NEGATIVE | Negative weight/status. |
| SCALE_OUT_OF_RANGE | Reading outside configured range. |
| PRINTER_OUT_OF_PAPER | Printer reports paper issue. |
| PRINTER_COVER_OPEN | Printer cover/status error. |
| PRINTER_ENCODING_ERROR | Receipt encoding/rendering failure. |
| PRINTER_WRITE_FAILED | Transport write failed. |
| DRAWER_OPEN_FAILED | Drawer trigger failed. |
| DISPLAY_WRITE_FAILED | Display transport/render failure. |
| HW_REVOKED | Device revoked. |
| HW_CONFIG_INVALID | Invalid device configuration. |

# 32. Reconnection & Retry Policy

Use exponential backoff with jitter, e.g. 1s, 2s, 4s, 8s, capped at a configured maximum.

Reset retry counter after stable successful connection.

Do not retry malformed protocol commands indefinitely.

Do not retry printer jobs automatically if doing so could produce duplicate physical prints unless the transport provides an acknowledgement model; expose manual retry instead.

Scale continuous read streams may reconnect automatically.

Payment operations must use payment-service idempotency, not hardware retry semantics.

# 33. Printer Duplicate-Print Safety

Printing is inherently difficult to make exactly-once because a connection can fail after the printer physically receives bytes. Therefore:

Each print attempt has a local print_job_id.

On uncertain completion, UI shows UNKNOWN/REQUIRES_RETRY rather than assuming failure.

Automatic retry is disabled for uncertain receipt jobs unless the printer/transport provides reliable job acknowledgement.

Manual retry is allowed and is an audited reprint if the invoice was already finalized.

Financial state never depends on print acknowledgement.

# 34. Scale Duplicate/Replay Safety

Every accepted stable weight has a local capture ID.

POS associates the capture ID with cart line/version.

Re-reading a scale creates a new capture ID.

An old capture cannot silently replace a newer confirmed weight.

At finalization, the quantity sent to the backend is validated against the current cart line and product/UOM rules.

# 35. Customer Display Consistency

Display is derived from current POS presentation state.

Display update failures do not mutate the authoritative cart.

During payment, the displayed amount must come from the same calculated quote shown to the cashier.

After finalization, display may show completion/thank-you state but must not reveal internal audit information.

# 36. Hardware Configuration Versioning

Every configuration has a version.

Changing baud rate, protocol profile, paper width, encoding mode, stability rules or device assignment increments configuration version.

Local Flutter cache stores the active version.

Server configuration wins for authoritative provisioning.

Config changes are auditable.

Rollback to a prior known-good configuration is supported for authorized operators.

# 37. Localization & Accessibility

Hardware setup screens support Tamil and English.

Use clear icons plus text for connection state.

Do not rely on color alone to communicate errors.

Scale weight must use large, readable typography.

Printer/scale failure messages should state the next action.

Touch targets must be suitable for shop-floor use.

# 38. Performance Targets

| Operation | Target |
| --- | --- |
| Scanner decode to POS event | p95 < 150 ms after complete scan input |
| Stable scale event after stability threshold | < configured stability window + 250 ms processing |
| Printer rendering | < 500 ms typical before transport |
| Printer command start | < 500 ms after print request |
| Hardware screen state update | < 200 ms typical |
| Reconnect initiation | < 1 s after detected disconnect |
| Diagnostic test | < 5 s typical excluding operator action |

Actual targets depend on hardware, OS and transport. Measurements must be collected on supported production devices.

# 39. Observability

Metrics by tenant/terminal/hardware type, avoiding excessive high-cardinality raw serial numbers.

Connection success/failure count.

Reconnect count.

Scale stable/unstable/error count.

Printer success/failure/uncertain count.

Scanner unknown-barcode count.

Hardware operation latency.

Firmware/model inventory where available.

Diagnostic failures.

Correlate hardware events to request ID, transaction ID or print_job_id where applicable.

# 40. Testing Strategy

## 40.1 Unit tests

Scanner terminator/prefix/suffix parsing.

Barcode normalization.

Scale frame parsing.

Checksum validation.

Unit conversion.

Stability filter.

Overload/negative/error handling.

ESC/POS command generation.

Tamil rendering path.

Cash drawer command generation.

Reconnect backoff.

Printer duplicate-print safety.

## 40.2 Integration tests

USB/Bluetooth/serial adapter integration using mocks or hardware test rigs.

POS + scale + invoice calculation.

POS + printer after successful finalization.

Printer failure after finalization.

Device revocation.

Offline hardware capture and sync.

Payment display with verified payment state.

## 40.3 Physical hardware test matrix

| Hardware | Tests |
| --- | --- |
| Scanner | rapid scans, unknown code, malformed input, disconnect/reconnect |
| Scale | zero, tare, stable, motion, overload, negative, max range, disconnect |
| Printer | Tamil, GST receipt, QR, barcode, paper-out, cover-open, disconnect, uncertain write |
| Drawer | open, failure, unauthorized user |
| Display | normal update, long text, disconnect/reconnect |
| Payment display | QR rendering, expiry, reconnect, no false paid state |

## 40.4 Golden tests

English receipt.

Tamil receipt.

Mixed Tamil/English product names.

GST split tax receipt.

Discount receipt.

Khata receipt.

UPI receipt.

Return/credit note representation.

# 41. Chaos & Failure Tests

Kill Flutter process during scale read.

Disconnect USB during printer write.

Disable Bluetooth while connected.

Restart printer during print.

Unplug scale during stable-reading wait.

Generate malformed serial frames.

Cause checksum errors.

Force network loss during finalized invoice flow.

Restart backend after device API request.

Revoke device while offline, then reconnect.

Simulate duplicate printer retry.

Simulate payment QR/display failure.

# 42. Operational Runbooks

| Incident | Operator action |
| --- | --- |
| Scale disconnected | Check power/cable/Bluetooth; run diagnostics; use authorized manual weight if policy allows. |
| Printer not printing | Check paper/cover/power/connection; retry print; use WhatsApp fallback. |
| Unknown barcode | Search product by Tamil/English/alias; escalate master-data issue. |
| Repeated protocol errors | Stop device, verify profile/baud/protocol, run diagnostics. |
| Device revoked | Admin/IT re-provisions or replaces device; no financial bypass. |
| Customer display failed | Continue POS; use cashier display. |
| Drawer failed | Continue cash sale; record/verify physical drawer operation during EOD. |
| Uncertain print | Do not create another invoice; inspect printer and use audited reprint. |

# 43. API / DB / Application Traceability

| Requirement | Primary source |
| --- | --- |
| Hardware identity/RBAC | Document 01 + 03 |
| Device API | Document 02 + 03 |
| Flutter adapter | Document 04 |
| Offline capture | Document 05 |
| Stock/weight impact | Document 06 |
| Payment/Khata/accounting impact | Document 07 |
| POS finalization boundary | Document 08 |
| Physical I/O | Document 09 |

# 44. Docker/DevOps Requirements

Backend API, worker and infrastructure run as containers as defined by the platform architecture.

Hardware access remains on Flutter/native OS layer; Docker must not be used as a workaround for mobile peripheral permissions.

Development can provide simulated hardware services/mocks inside Docker for CI.

Hardware protocol fixtures are version-controlled test assets.

CI must run adapter unit tests without requiring physical devices.

Optional hardware-in-the-loop test environment runs separately with controlled physical rigs.

# 45. Simulated Hardware Services

For local development and CI, provide deterministic simulators for scanner, scale and printer.

hardware-simulator/
  scanner/
    emit_scan
  scale/
    stable_weight
    unstable_weight
    overload
    disconnect
  printer/
    accept_job
    fail_job
    uncertain_job
  display/
    render

Simulator responses must reproduce success, timeout, malformed input, disconnect and error states.

Simulators must never connect to production payment or financial databases.

Test fixtures should include Tamil product names and realistic decimal weights.

# 46. Data Retention & Privacy

Retain device configuration and audit according to operational retention policy.

Do not retain raw scanner input longer than needed unless diagnostic policy requires it.

Raw scale frames should normally be diagnostic-only and short-retention.

Receipt copies follow invoice/document retention policy.

Hardware logs must be scrubbed of tokens, payment secrets and unnecessary PII.

# 47. Firmware & Updates — Future Extension

Firmware updates are outside the v1.0 POS hardware control path.

If introduced, firmware must be signed, version-checked and rollback-capable.

Only authorized administrators may initiate updates.

Power-loss recovery must be designed before enabling production firmware updates.

Firmware version is informational in v1.0 unless a device compatibility rule explicitly requires it.

# 48. Acceptance Criteria

Scanner reliably captures supported barcodes and never creates products automatically.

Scale captures stable weights with configured protocol and unit conversion.

Unstable/overload/negative readings cannot be accepted as valid automatic sale weights.

Authorized manual weights are audited.

Printer produces correct English and Tamil receipts for all supported profiles.

Printer failure never rolls back a finalized invoice.

Uncertain printer outcomes do not create duplicate invoices.

WhatsApp fallback can be offered after printer failure where configured.

Cash drawer can be triggered only by authorized operations.

Customer display failure does not block billing.

Payment/QR display never determines payment success.

Hardware pairing is tenant/device scoped and audited.

Revoked devices cannot perform authoritative operations.

Offline hardware events survive app restart where required.

All hardware adapters have deterministic mocks.

Physical test matrix passes for each supported hardware profile.

Docker-based CI passes all simulator/adapter tests.

All critical hardware failures have actionable operator diagnostics.

# 49. Implementation Checklist

Define supported scanner models/protocols.

Define supported scale models/protocol profiles.

Define supported printer models/paper widths.

Implement HardwareManager lifecycle.

Implement device registry/pairing state.

Implement scanner adapters.

Implement scale adapters and stability filter.

Implement printer adapters and ESC/POS renderer.

Implement Tamil receipt rendering.

Implement cash drawer adapter.

Implement customer display adapter.

Implement payment/QR display abstraction.

Implement diagnostics UI.

Implement reconnect policies.

Implement hardware permissions handling.

Implement local hardware configuration storage.

Implement audit events.

Implement hardware simulators.

Implement physical hardware test matrix.

Integrate with POS Document 08.

Integrate with offline Document 05.

Validate Docker/CI simulator environment.

Prepare operational runbooks.

# 50. Non-Negotiable Hardware Rules

Never allow physical hardware success to be the authority for a financial transaction.

Never allow printer failure to roll back a finalized invoice.

Never trust a scale stability flag without backend/POS quantity validation.

Never accept negative, overload or invalid weight as a normal sale quantity.

Never auto-create products from scanner input.

Never use hardware adapters directly from accounting/inventory code.

Never perform Bluetooth/USB/RS-232 physical I/O from backend Docker containers.

Never retry an uncertain printer job automatically in a way that can silently duplicate a receipt.

Never expose payment secrets in hardware logs.

Never let customer display/payment display determine payment settlement.

Never allow revoked devices to perform authoritative operations.

Never bypass audit for manual weight, reprint, drawer operation or hardware configuration.

Never hardcode a single serial protocol configuration for all scales.

Never assume all printers support Unicode/Tamil natively; use a tested fallback rendering strategy.

# 51. End-to-End Hardware Scenarios

## 51.1 Barcode cash sale

Scanner reads barcode.

POS resolves product.

Cashier confirms quantity.

POS calculates quote.

Cash accepted.

Server finalizes invoice.

Post-commit printer prints.

Drawer kick occurs if configured.

## 51.2 Loose feed by scale

Cashier selects loose product.

Scale streams readings.

Stable reading accepted.

POS associates capture ID.

Quote calculated.

Payment completed.

Server finalizes stock/accounting.

Receipt prints.

## 51.3 Scale disconnected

Disconnect detected.

POS preserves cart.

Cashier reconnects or uses authorized manual weight.

Manual entry reason recorded.

Finalization performs normal server validation.

## 51.4 Printer fails after sale

Invoice commits.

Printer write fails.

Invoice remains FINALIZED.

Cashier retries or shares via WhatsApp.

Audit records print/share outcome.

## 51.5 Uncertain print

Printer transport times out after bytes may have been delivered.

POS marks print outcome UNKNOWN.

No new invoice is created.

Operator checks physical printer.

Operator uses controlled reprint if required.

## 51.6 Offline sale

Scanner/scale work locally.

Sale is stored in SQLCipher envelope.

Local receipt follows offline policy.

Network returns.

Envelope uploads idempotently.

Server reconciles inventory/credit/price policy.

Authoritative result is returned.

# 52. Example Hardware Configuration

{
  "terminal": "POS-01",
  "scanner": {
    "type": "USB_HID",
    "suffix": "\n",
    "debounce_ms": 250
  },
  "scale": {
    "type": "RS232",
    "profile": "VENDOR_PROFILE_A",
    "unit": "kg",
    "stability_duration_ms": 800,
    "allowed_variance": 0.020
  },
  "printer": {
    "type": "USB_ESCPOS",
    "paper_width_mm": 80,
    "tamil_render_mode": "BITMAP",
    "auto_retry_uncertain": false
  },
  "drawer": {
    "mode": "PRINTER_KICK",
    "enabled": true
  }
}

This is illustrative configuration only. Actual protocol parameters are selected per supported device model.

# 53. Definition of Done

All supported hardware has an adapter contract.

Each adapter has unit tests and simulator fixtures.

Scale stability and error handling are verified.

ESC/POS receipt output is verified in physical tests.

Tamil receipt output is verified.

Printer failure isolation is verified.

Unknown/duplicate scanner behavior is verified.

Hardware configuration is tenant/device scoped and audited.

Revocation works server-side.

Offline behavior is tested across restart/reconnect.

POS Document 08 integration passes all hardware scenarios.

Operational diagnostics and runbooks are available.

CI runs hardware simulation tests inside Docker.

No P0 hardware/data-integrity defect remains.

# 54. Dependency & Change Control

This document depends on Documents 01–08 and is normative for hardware boundaries used by Documents 10–20. Changes to hardware protocols, device capabilities, receipt rendering, scale precision or payment-display behavior must record compatibility impact, affected terminals/models, migration/configuration changes and test evidence.

End of Document 09 — Complete Hardware Integration Specification v1.0.